Plant Research Findings for 
Optimal Temperature, Soil Moisture, 
LED Light Pattern & Musical Stimulation.

Contents

Plant Types:	3

Temperature:	3

Soil Moisture:	4

LED Light Patterns/Colour:	5

Musical Stimulation:	5

Summary	6

# Plant Research Findings for 
Optimal Temperature, Soil Moisture, 
LED Light Pattern & Musical Stimulation.

# Plant Types:

Plant 1: Rubella Peperomia
Initial Height: 4”
Initial Width: 4.5”

Plant 2: Peperomia Hope
Initial Height: 5”
Initial Width: 6”

Plant 3: Peperomia Marble
Initial Height: 6.5”
Initial Width: 9”

Plant 4: Cupid Peperomia
Initial Height: 6”
Initial Width: 9”

Temperature:

Output: Reading levels are typically read in degrees. 
Optimal temperature to maintain for plants: 18°C – 29°C
Specs:

- Usable with 3.0V to 5.5V power/data

- ±0.5°C Accuracy from -10°C to +85°C

- Usable temperature range: -55 to 125°C (-67°F to +257°F)

- 9 to 12 bit selectable resolution

- Uses 1-Wire interface- requires only one digital pin for communication.

- Unique 64 bit ID burned into the chip

- Multiple sensors can share one pin

- Temperature-limit alarm system

- Query time is less than 750ms.

- 3 wires interface:

- Type A

- Red wire – VCC

- Black wire – GND

- Yellow wire – DATA

- Type B (Recently there's a series of sensors using this pin mapping. Sorry for the inconvenience.)

- Red wire – VCC

- Yellow wire – GND

- Green wire – DATA

- Stainless steel tube 6mm diameter by 35mm long

- Cable diameter: 4mm (0.16)

- Length: 95cm (37.4")

Soil Moisture:

Output: Reading levels are typically read in percentages of RH (relative humidity).
Optimal soil moisture to maintain for plants: 50-70%, trigger point for irrigation should be around 40-45%.
Specs:

- Supports 3-Pin Gravity Sensor interface.

- Analog output

  - Applications:

    - Garden plants

    - Moisture detection

    - Intelligent agriculture

  - Technical Specifications:

    - Operating Voltage: DC 3.3-5.5V

    - Output Voltage: DC 0-3.0V

    - Interface: PH2.0-3P

    - Size: 99x16mm/3.9×0.63″

      - More information:

      - This capacitive soil moisture sensor measures soil moisture levels by capacitive sensing, rather than resistive sensing like other types of moisture sensor. It is made of corrosion-resistant material giving it long service life.

      - insert it into the soil and impress your friends with the real-time soil moisture data.

      - This module includes an on-board voltage regulator which gives it an operating voltage range of 3.3 ~ 5.5V.

      - This sensor is compatible with DFRobot 3-pin “Gravity” interface, which can be directly connected to the Gravity I/O expansion shield.

LED Light Patterns/Colour:

Angled lighting: Angled lighting promotes equal height and width growth for plants.
Colours: Blue, Red & White
Duration: 12 to 16 hours per day, can be indirect
Brightness Levels: moderate to light intensity 40-50%
Specs:

- Product name: 2 heads led grow lamp with timer

- Input Voltage: 100-240

- Output Voltage: 5V

- LED Quantity: 40pcs

- Color of lamp beads: Full Spectrum

- Material: Aluminum

- Power cable: about 2.1m

- Frequency: 50-60Hz

  - More Information:

    - 3 Lighting Modes: warm white, red+blue, warm white+blue+red lighting modes options

    - Timing Modes: 6H/12H/16H automatic memory cycle lighting everyday

    - 5 Dimming Levels: Brightness adjustable range is 20%-100%

# Musical Stimulation:

Music Genre: Musical stimulation of jazz or classical music with a frequency of 115-220Hz is optimal for plant growth. Time of day to play music for plants: Morning to early afternoon.
Durations: 1-3 hours 
Volume: Moderate 
Things to avoid: Playing music at night, it is typically the plants resting time. Playing music too loud can cause negative effects.
Playlist chosen for plants: https://open.spotify.com/playlist/2QqHZfdePzosdFgvHdz52h?go=1&sp_cid=e7c8ed27eafb47643f2c77b5d7c16bf0&utm_source=embed_player_p&utm_medium=desktop&nd=1&dlsi=814474ac3c544b78

# Summary

Temperature:

- Optimal Temperature: 18-29C

Soil Moisture:

- Optimal Soil Moisture: 50-70%

- Irrigation Trigger Point: 40-45%

LED Light Pattern:

- LED Light Colour: Combination of Red and Blue

- LED Timer: 12hrs to 16 hrs

- LED distribution: Angled Lighting

- LED light intensity: 40-50%

Musical Stimulation:

- Musical Genre: Jazz or Classical

- Musical Timer:1 to 3 hrs early afternoon

- Musical Volume: Moderate

- Musical Hertz: 115-220Hz